| Project Title: | Multi-Panel Electronics Monitoring System with Web Based Dashboard |
| --- | --- |
| Student: | Chan Zi Min Edwin (UP2517673) |
| Course: | UOP BSc Computer Science |
| Project code: | PJE40 for Engineering project |
| Supervisor: | Dr Loo Poh Kok |
| Date: | May 2026 |

Declarations (Please tick)

|  | I give permission for this document to be made available to other students as examples of previous work. (optional). |
| --- | --- |
|  | I confirm that I have read and understood the University Rules in respect of plagiarism and student misconduct. |
|  | I declare that this work is entirely my own. Each quotation or contribution cited from other work is fully referenced. |

Date: ________________

[left intentionally blank]

Acknowledgements – on a new page

on a new page and centred

50 to 100 words

should not appear in contents list

can be coloured if you wish

Abstract – on a new page

on a new page and centred

about 100 words

should not appear in contents list

can be coloured if you wish

Table of Contents – on a new page

Table of Figures – on a new page

Table of Tables – on a new page

# Introduction

## Electronics Enclosure Monitoring System

Electronics enclosures house crucial components in server rooms, industrial sites, and remote installations. These enclosures contain sensitive equipment susceptible to damage from environmental factors, most notably excessive heat. Prolonged exposure to high temperatures reduces the operational lifespan of electronic components, leads to unpredictable failure, and results in costly unplanned maintenance (Lv, Chu and Wang, 2022). Traditionally, monitoring has relied on manual inspection and a reactive approach where faults are only discovered after damage has occurred.

The rise of Internet of Things (IoT) technology has created an opportunity to replace manual checks with continuous automated monitoring. Low-cost microcontrollers such as the ESP32 can read environmental sensors and transmit data over lightweight communication model protocols like MQTT to a central server for processing and visualization.

This project aimed at developing a multi-panel electronics enclosure monitoring system with automated cooling, critical condition alerts, and a web-based dashboard for real-time status and historical trend analysis. The system was designed to demonstrate how open-source, low-cost technologies could deliver a practical monitoring solution for small-to-medium-scale deployments.

## Project Aim and Objectives

The aim of this project was to develop a multi-panel electronics enclosure monitoring system with automated cooling, alert notifications, and a web-based dashboard for real-time monitoring and historical trend analysis.

Objectives:

- Research existing IoT monitoring solutions and evaluate suitable communication protocols, backend frameworks, databases, and frontend technologies.

- Design a system architecture using MQTT as the communication protocol between sensor nodes and a central backend server.

- Develop ESP32 firmware to read temperature and humidity sensors, control cooling automatically, and publish data via MQTT.

- Configure a self-hosted Mosquitto MQTT broker with password authentication for secure message routing.

- Implement a FastAPI backend server to subscribe to MQTT messages, process sensor data, and store readings in a PostgreSQL database.

- Design and populate a PostgreSQL database to maintain current panel state and historical sensor logs.

- Build a React-based web dashboard with Chart.js for real-time panel monitoring, historical trend visualisation, and remote system control.

- Implement an alert system to detect and log critical conditions (example: high temperature, door timeout, and security breach).

- Conduct end-to-end testing across the full system pipeline and evaluate against defined requirements.

## Project constraints

The project was subject to the following constraints:

- Time - The submission deadline of 23 June 2026 was fixed, limiting the scope of features that could be implemented.

- Budget - Self-funded with consumer-grade components. Only one physical ESP32 node was built; the remaining nine panels were simulated.

- Hardware - The ESP32 microcontroller only supports 2.4 GHz Wi-Fi, restricting deployment to compatible wireless environments.

- Network - All development and testing were conducted on a local home network using a mobile hotspot bridge. No cloud infrastructure was used.

- Development environment - All work was performed on personal equipment with no access to dedicated server hardware or university lab resources.

- Scope - Features such as RFID access control, message alert notifications, and log for predictive maintenance were deferred to future work to prioritise core monitoring functionality.

## Log of risks

Table 1.1 Risk Log

| # (S/N) | Risk | Likelihood | Impact | Mitigation | Outcome |
| --- | --- | --- | --- | --- | --- |
| 1 | Hardware component failure | Low | High | Spare components ordered; 9 of 10 panels simulated to reduce hardware dependency. | Did not occur |
| 2 | Data loss | Medium | High | Version control via GitHub; PostgreSQL database persistence enabled. | Did not occur |
| 3 | Scope creep | Medium | Medium | Core features prioritised; non-essential features deferred to future work. | Mitigated successfully |
| 4 | Component integration complexity | Medium | Medium | Phased development approach; each component tested independently before integration. | Mitigated successfully |
| 5 | Network connectivity failure | Medium | Medium | Network topology documented; mobile hotspot configured as bridge. | Occurred - resolved |
| 6 | RFID module compatibility | Low | Medium | Early testing planned; compatible modules researched. | Not applicable - feature deferred |
| 7 | Personal illness or commitments | Low | High | Buffer time allocated in project schedule. | Occurred - resolved |
| 8 | Network failure during demonstration | Medium | Medium | Mobile hotspot backup prepared; video demonstration recorded as fallback. | Pending confirmation |

## Project deliverables

The following deliverables were produced upon completion of the project:

Artefacts:

- ESP32 firmware - sensor monitoring, automated fan control, keypad access control, alarm system (buzzer) and MQTT communication.

- Mosquitto MQTT broker - configured with password authentication and message persistence.

- FastAPI backend server - MQTT subscriber, PostgreSQL database integration, and REST API endpoints

- PostgreSQL database - three-table schema for current state tracking and historical event logging.

- React web dashboard - Chart.js visualisation, real-time status monitoring, CSV data Log export, and remote system reset.

Documentation:

- Final project report.

- System architecture diagrams.

- User guide and deployment instructions.

- Source code repository hosted on GitHub (refer to Appendix F).

- Video demonstration of the complete system (refer to Appendix G).

## Project Approach

### Project Management

The project followed a phased development approach. Early prototyping began with the ESP32 and Adafruit IO as a cloud-based MQTT platform to test sensor communication. However, limitations of the Adafruit platform - such as restricted message rates, limited control over data storage, and dependency on a third-party service - which prompted a shift toward a fully self-hosted stack. This experience directly informed the selection of Mosquitto, FastAPI, and PostgreSQL as replacements, providing full control over the system architecture.

Following the pivot, development was divided into sequential steps, each with clear completion criteria tested before progressing:

Table 1.2 Task List

| Step | Focus | Purpose |
| --- | --- | --- |
| 1 | ESP32 firmware | Hardware prototype, sensor integration, fan control, keypad access, alarm system, and MQTT publishing. |
| 2 | Mosquitto broker | Self-hosted MQTT broker with password authentication |
| 3 | Development environment | Python virtual environment and package installation |
| 4 | MQTT subscriber | Standalone proof-of-concept to verify broker communication |
| 5 | PostgreSQL database | Schema design, table creation. |
| 6 | FastAPI backend | MQTT ingestion, database writes, and REST API endpoints |
| 7 | React frontend | Web dashboard with visualisation, export, and remote control |

Progress was managed using a Gantt chart to track tasks, milestones, and deadlines (see Appendix A). Supervisor meetings were held regularly with Dr. Loo Poh Kok for progress reviews and guidance. Version control was managed through GitHub.

### Research

Research combined secondary and primary methods. Secondary research involved reviewing official documentation, online resources, and open-source repositories to evaluate suitable technologies. Primary research involved hands-on prototyping - initial development with Adafruit IO revealed limitations, leading to a pivot to a self-hosted Mosquitto architecture. Both platforms had been introduced during the IoT module. Findings are presented in Section 2.

## Legal, ethical, professional, social issues

Legal: All software tools and libraries used in the project were open-source or freely available, including Arduino IDE, Mosquitto, FastAPI, PostgreSQL, and React. No proprietary or licensed software was used. The system collected environmental sensor data only - no personal or identifiable data was stored; therefore, data protection regulations did not apply in the current scope. However, if the system were deployed in a workplace environment with user access logging, data protection compliance would need to be considered.

Ethical:

The project did not involve human participants, personal data collection, or animal testing. All data was generated by environmental sensors and simulated panels. An ethics certificate was obtained and is included in Appendix B.

Professional:

Development followed established software engineering practices including version control (GitHub), modular architecture, structured testing, and technical documentation. Industry-standard technologies were selected to align with professional development standards.

Social:

The system was designed to reduce the need for manual on-site inspection of electronics enclosures, improving workplace safety and operational efficiency. Automated alerts for critical conditions such as overheating and security breaches enable faster response times, reducing the risk of equipment damage and potential safety hazards.

## The report

The remainder of this report is structured as follows:

Section 2 presents a literature review of existing IoT monitoring solutions and evaluates the communication protocols, backend frameworks, database systems, and frontend technologies considered for this project.

Section 3 captures the functional and non-functional requirements of the system, including use cases and scope boundaries.

Section 4 describes the system design, covering the overall architecture, database schema, MQTT topic structure, API design, frontend layout, and ESP32 firmware logic.

Section 5 details the development process, walking through each implementation step from ESP32 firmware to the React web dashboard.

Section 6 presents the testing strategy, test plan, results, and evidence demonstrating that the system meets the defined requirements.

Section 7 concludes the report with an evaluation of whether the aim and objectives were met, a review of risks and project management, recommendations for future work, and a final reflection on the project.

# Literature Review

## Introduction

This section reviews the technologies and solutions relevant to the development of the project. It covers IoT monitoring systems, communication protocols, MQTT brokers, backend frameworks, database systems, and frontend technologies. Each section evaluates the available options and the justification for the technology selected.

## Research Strategy

Research was conducted using a combination of official documentation, online resources, open-source repositories, and hands-on prototyping. Sources included official documentation for each technology (Mosquitto, FastAPI, PostgreSQL, React, ESP32), developer community resources, and GitHub repositories. Technologies were evaluated based on suitability for IoT applications, ease of integration, community support, and relevance to industry practice. Initial hands-on experience with Adafruit IO during the IoT module also informed the evaluation while testing on the prototype, as its limitations provided first-hand evidence for the selection of a self-hosted architecture.

## IoT Monitoring Systems

In industrial and commercial environments, monitoring of equipment such as server rooms, telecommunications enclosures, and electrical panels has traditionally relied on manual inspection or proprietary monitoring solutions. These approaches are often costly, difficult to customize, and impractical for small-to-medium-scale deployments.

The emergence of IoT technology has introduced a more accessible alternative. IoT-based monitoring systems use low-cost microcontrollers and wireless connectivity to collect and transmit sensor data to a central server for processing and visualization. The ESP32, developed by Espressif Systems, is a low-cost, low-power system-on-chip with integrated Wi-Fi and Bluetooth capabilities, making it well-suited for IoT sensor applications (Espressif Systems, 2024).

IoT systems typically follow a layered architecture consisting of a perception layer for data collection, a network layer for data transmission, and an application layer for delivering services to end users (Banaamah and Ahmad, 2024). This project adopted a similar architecture: the ESP32 served as the perception layer, MQTT over Wi-Fi as the network layer, and the FastAPI backend with React dashboard as the application layer.

During the prototyping phase, Adafruit IO - a cloud-based IoT platform - was initially tested. While it provided a simple interface for publishing and visualizing sensor data, it imposed limitations including restricted message rates, limited data retention, and dependency on an external service. These constraints led to the decision to implement a fully self-hosted architecture using Mosquitto, FastAPI, and PostgreSQL, giving complete control over data storage, message handling, and system configuration.

## Communication Protocols for IoT

A key design decision for the project was selecting the communication protocol between the ESP32 sensor nodes and the backend server. Three protocols were evaluated:

MQTT is an OASIS standard messaging protocol designed for IoT. It uses a lightweight publish/subscribe model where clients publish messages to topics, and a central broker routes them to subscribers (OASIS, 2014). MQTT was designed for devices with small code footprints and minimal network bandwidth (mqtt.org, no date).

HTTP/REST follows a request/response model where the client must actively poll the server, or the server must be configured to accept incoming data. Each request carries headers that add overhead compared to MQTT's minimal packet structure. For continuous sensor data published at regular intervals, the polling approach introduces unnecessary latency and bandwidth consumption.

WebSockets provide a persistent, full-duplex connection between client and server, enabling real-time bidirectional communication. While effective for browser-to-server applications, maintaining persistent connections on constrained microcontrollers increases power consumption and firmware complexity compared to MQTT's lightweight model.

The publish/subscribe model is illustrated in Figure 2.1.

Figure 2.1 MQTT publish/subscribe model.

The following table summarises the communication protocol:

Table 2.1 Communication Protocol Comparison

| Feature | MQTT | HTTP/REST | WebSocket |
| --- | --- | --- | --- |
| Model | Publish/subscribe | Request/response | Full-duplex |
| Transport | TCP | TCP | TCP |
| Overhead | Low (minimal headers) | High (HTTP headers) | Medium |
| Broker/routing | Yes (central broker) | No | No |
| Asynchronous | Yes (non-blocking) | No (waits for response) | Yes |
| Decoupling | Yes (via broker) | No (direct connection) | No (direct connection) |
| Suitability for constrained devices | High | Low | Medium |
| One-to-many distribution | Built-in via broker | Requires additional architecture | Requires additional architecture |

Justification: MQTT was selected for this project for three key reasons. First, it is asynchronous - the ESP32 publishes sensor data without waiting for a response, allowing the device to continue operating without blocking. Second, it is decoupled - the publish/subscribe model means the ESP32 does not need to know which systems are consuming its data, and the backend can be restarted or replaced without affecting the device. Third, it is lightweight - MQTT's minimal packet headers and small code footprint make it well-suited to the ESP32's constrained resources (OASIS, 2014). These three qualities made MQTT the most suitable protocol for a multi-panel monitoring system where multiple sensor nodes broadcast data to a central backend.

## MQTT Broker Comparison

Three MQTT broker platforms were evaluated during the research and prototyping phase:

Adafruit IO is a cloud-based IoT platform that provides MQTT messaging, data storage, and a built-in dashboard. It was tested during early prototyping with the ESP32. However, Adafruit IO's feed structure did not support hierarchical topic organisation - feeds operated at root level rather than being grouped by panel (e.g., panel1/temperature). Additionally, the free tier was limited to 10 feeds, which was insufficient for a multi-panel system requiring five data types per panel across 10 panels. These limitations, combined with dependency on an external service, made Adafruit IO unsuitable for the project.

HiveMQ is a commercial MQTT broker designed for enterprise-scale deployments. It offers advanced features such as clustering, extensive monitoring, and a managed cloud option. However, the full feature set requires a paid licence, and the complexity of the platform exceeded the requirements of this project.

Eclipse Mosquitto is an open-source MQTT broker maintained by the Eclipse Foundation. It is lightweight, supports MQTT versions 5.0, 3.1.1, and 3.1, and is suitable for use on devices ranging from low-power single-board computers to full servers (Eclipse Foundation, no date). Mosquitto supports password-based authentication, message persistence, and configurable logging.

Mosquitto was selected for this project because it provided full control over the broker configuration as a self-hosted solution, it is open-source and free to use, it is lightweight enough to run alongside the backend on a single development machine, and it is widely documented with strong community support.

## Backend Framework Selection

The backend framework was selected based on industry relevance and suitability for the project requirements. FastAPI is a modern Python web framework designed for building APIs. It provides automatic interactive API documentation via Swagger UI, enabling developers to test REST API endpoints directly from the browser without additional tools (FastAPI, no date). FastAPI was chosen for this project because it is widely adopted in industry, it integrates well with Python-based MQTT libraries such as paho-mqtt, and the built-in Swagger UI significantly simplified API testing during development.

## Database Selection

The database was selected based on industry relevance and suitability for structured sensor data storage. PostgreSQL is an open-source relational database system known for its reliability, data integrity, and support for complex queries (PostgreSQL, no date). It enforces referential integrity through foreign keys, ensuring consistency between related tables. PostgreSQL was chosen for this project because it is widely adopted in industry, it supports the relational structure needed for linking panels to their sensor readings and alerts, and it is well-supported by Python through the psycopg2 library.

## Frontend Framework Selection

The frontend framework was selected based on industry relevance, suitability for building an interactive dashboard, and prior experience. React is an open-source JavaScript library developed by Meta for building component-based user interfaces (React, no date). The developer had prior experience with React from the Software Engineering Theory and Practice module, which reduced the learning curve. React was chosen for this project because it is widely adopted in industry, its component architecture supports separation of concerns between different parts of the dashboard, and it integrates well with Chart.js for data visualisation through the react-chartjs-2 wrapper library.

## Summary

This section reviewed the technologies relevant to the project and provided justification for each selection. IoT monitoring systems were examined to understand common architectures and identify the opportunity for a low-cost, self-hosted solution. MQTT was selected as the communication protocol for its asynchronous, decoupled, and lightweight characteristics. Mosquitto was chosen as the MQTT broker after first-hand testing of Adafruit IO revealed structural and scalability limitations. FastAPI, PostgreSQL, and React were each selected based on industry relevance, project suitability, and in the case of React, prior experience. Thus, these technologies formed a fully open-source, self-hosted stack providing complete control over the system architecture.

# Requirements Capture (Analysis)

## Introduction

This section defines the functional and non-functional requirements of the monitoring system. Requirements were derived from the project brief, supervisor guidance, domain research, and first-hand experience during prototyping. Each requirement is assigned a priority and is later mapped to test cases in Section 6.

## Functional Requirements

The following functional requirements were identified for the system:

Table 3.1 Functional Requirements Specification

| ID | Requirement | Priority |
| --- | --- | --- |
| FR1 | The system shall read temperature and humidity from sensors at 10-second intervals | Must |
| FR2 | The system shall publish sensor data to an MQTT broker | Must |
| FR3 | The system shall activate fan cooling when temperature reaches 45°C and deactivates at 38°C (hysteresis) | Must |
| FR4 | The system shall trigger a critical alarm when temperature exceeds 75°C | Must |
| FR5 | The system shall detect door open/close events via reed switch | Must |
| FR6 | The system shall trigger a timeout alarm if the door remains open for 60 seconds without password entry | Must |
| FR7 | The system shall trigger a security alarm after three incorrect password attempts | Must |
| FR8 | The system shall store current panel state and historical sensor readings in a database | Must |
| FR9 | The system shall support monitoring of up to 10 panels (1 physical, 9 simulated) | Must |
| FR10 | The system shall display all panel statuses on a web dashboard | Must |
| FR11 | The system shall display historical temperature and humidity charts | Should |
| FR12 | The system shall allow remote system reset via the dashboard | Should |
| FR13 | The system shall provide CSV export of “sensor readings” and “alerts” | Should |
| FR14 | The system shall indicate online/offline status per panel | Should |
| FR15 | The system shall display colour-coded status indicators for critical conditions | Should |

## Non-Functional Requirements

The following non-functional requirements define the quality attributes of the system:

Table 3.2 Non - Functional Requirements Specification

| ID | Requirement | Category |
| --- | --- | --- |
| NFR1 | The dashboard shall update panel data within 10 seconds of new sensor readings | Performance |
| NFR2 | The system shall support monitoring of 10 panels simultaneously | Scalability |
| NFR3 | The backend shall remain operational and serve last-known data if the MQTT connection drops | Reliability |
| NFR4 | MQTT communication shall use password-based authentication | Security |
| NFR5 | The dashboard shall display a warning banner when the backend is unreachable | Usability |
| NFR6 | The dashboard shall use colour-coded indicators matching firmware thresholds (green, amber, red) | Usability |
| NFR7 | The system shall operate on a local network without cloud dependency | Deployment |
| NFR8 | The frontend shall provide a basic responsive layout for smaller screens | Usability |

## Use Cases

The following use cases describe the key interactions between the operator and the system:

Table 3.3 Use Cases Specification

| UC | Actor | Description |
| --- | --- | --- |
| UC1 | Operator | View a list of all panels in the sidebar with online/offline status indicators |
| UC2 | Operator | Select a panel from the sidebar to view detailed status including temperature, humidity, door, fan, and alarm status |
| UC3 | Operator | View historical temperature and humidity trends for the selected panel over the past 30 days |
| UC4 | System | Detect a critical condition (high temperature, door timeout, or security breach) and create an alert record in the database |
| UC5 | Operator | Perform a remote system reset on a panel via the dashboard |
| UC6 | Operator | Export sensor readings or alerts as a CSV file for a selected panel based on past records |
| UC7 | On-site technician | Enter password on keypad to authenticate that the person opening the door is authorised personnel, preventing a security alarm from triggering |
| UC8 | On-site technician | Acknowledge a critical temperature alarm by double-pressing the reset button to silence the buzzer while performing on-site maintenance |

## Scopes of Boundaries

The following table defines what was included within the project scope and what was deliberately excluded:

Table 3.4 System Scope Specifications

| In Scope | Out of Scope (Future Work) |
| --- | --- |
| 1 physical ESP32 panel with 9 simulated panels | RFID access control: card scan combined with passcode entry to authenticate individual personnel and log access records |
| Temperature and humidity monitoring with automated fan control | Message alert notifications (email/SMS): notify operators when critical conditions occur or unauthorised personnel open the door, enabling immediate response without monitoring the dashboard |
| Door access control with keypad authentication | Predictive maintenance prompts: the system does not prompt operators to schedule maintenance based on recurring patterns |
| Alert system for critical temperature, door timeout, and security breach | Docker containerisation |
| Self-hosted Mosquitto MQTT broker with password authentication | Cloud deployment (Linode/AWS) |
| FastAPI backend with REST API and MQTT subscriber | TLS/SSL encryption for MQTT |
| PostgreSQL database for current state and historical logging | Alert recovery logging: the current system records when alerts are triggered but does not record when errors are resolved |
| React web dashboard with Chart.js visualisation | User authentication for the dashboard: without login, any user with the URL could access the system and perform actions such as repeated system resets |
| CSV export of sensor readings and alerts | Automated testing framework |
| Remote system reset via dashboard | Mobile-first responsive redesign |

The system was developed as a working prototype for demonstration and evaluation purposes. All development and testing were conducted on a local network. Cloud -based deployment and Docker containerisation were considered but placed outside the current scope and are recommended as future enhancements (see Section 7).

## Summary

This section defined the functional and non-functional requirements of the monitoring system, identified key use cases for both remote operators and on-site technicians, and established clear scope boundaries. A total of 15 functional requirements and 8 non-functional requirements were identified, all of which were implemented and are evaluated against test cases in Section 6.

# Design

## Introduction

This section describes the design of the monitoring system, covering the overall architecture, data flow, database schema, MQTT topic structure, REST API endpoints, frontend layout, and ESP32 firmware logic. Design decisions reference the technology selections justified in Section 2.

## System Architecture

The system follows a layered architecture consisting of five main components connected through two communication protocols. The architecture is illustrated in Figure 4.1.

Figure 4.1 System architecture diagram.

At the perception layer, the ESP32 microcontroller reads temperature and humidity data from the DHT11 sensor and monitors door, fan, and alarm status. Sensor data is published to the Mosquitto MQTT broker over Wi-Fi using the PubSubClient library.

The Mosquitto broker acts as the central message router, receiving published messages from the ESP32 and delivering them to subscribed clients. Password-based authentication is enforced for all connections.

The FastAPI backend operates as both an MQTT subscriber and an HTTP server. A background thread subscribes to all panel topics via paho-mqtt and writes incoming data to the PostgreSQL database. The main thread serves REST API endpoints to the frontend via uvicorn on port 8000.

PostgreSQL stores data across three tables: a rolling current-state table for each panel, an append-only sensor readings log, and an append-only alert log. The database acts as a shared data store between the MQTT subscriber and REST API by providing loose coupling between the two subsystems.

The React frontend polls the REST API every 5 seconds and renders the dashboard. React never communicates with PostgreSQL or Mosquitto directly, all data flows through the FastAPI backend.

This architecture provides fault tolerance through loose coupling. If the MQTT connection drops, the REST API continues serving last-known data from PostgreSQL. If the frontend is unavailable, the backend continues receiving and storing sensor data.

## Data Flow

The system supports two directional data flows: sensor data flowing inward from the ESP32 to the dashboard, and control commands flowing outward from the dashboard to the ESP32.

Figure 4.2 Data flow diagram.

Sensor Data Flow (inbound):

The ESP32 reads temperature and humidity from the DHT11 sensor every 10 seconds.

Each reading is published as a separate MQTT message to the Mosquitto broker (e.g., panel/1/temperature, panel/1/humidity).

The FastAPI backend receives the message via the paho-mqtt on_message callback

The callback writes to PostgreSQL: an INSERT into sensor readings (append-only log) and an UPDATE to the panels table (current state).

- If the reading triggers an alert condition (temperature ≥ 75°C), an INSERT is made into the alerts table.

- The React dashboard polls GET /panels every 5 seconds and renders the updated data.

- Control Command Flow (outbound):

- The operator clicks the "Reboot" button on the dashboard then Clicked ''Ok''

- React sends a POST /panels/{id}/control/reset request to FastAPI

- FastAPI publishes a "RESET" payload to panel/{id}/control/reset via MQTT

- Mosquitto route/sends the message to the ESP32

- The ESP32 executes performFullSystemReset()

The two flows are loosely coupled through PostgreSQL. The inbound flow writes to the database, and the dashboard reads from it. The only point where the two flows connect directly is the reset command, where FastAPI publishes back to the MQTT broker.

## Database Design

The PostgreSQL database fyp_monitoring was designed with three tables, each serving a distinct purpose. The Entity Relationship Diagram is shown in Figure 4.3.

Figure 4.3 Entity Relationship Diagram.

The panels table stores the current state of each panel. It is updated in place on every incoming MQTT message, overwriting previous values. This provides the dashboard with a single query point for live status without scanning the full history log.

The sensor_readings table is an append-only event log. Each MQTT message creates a new row with only one data column populated (temperature or humidity), since the ESP32 publishes each data type as a separate message. This record and preserves the complete history log for trend analysis.

The alerts table is an append-only event log that records alert events when critical conditions are detected. Each alert logs includes the type, severity, triggering value, and a timestamp.

Both sensor_readings and alerts reference the panels table through a foreign key (panel_id), enforcing referential integrity. The relationship is one-to-many: one panel can have many readings and many alerts.

This design separates current state from historical records. The panels table answers "what is the current status?" while sensor_readings and alerts answer "what was the status in the past like?".

## MQTT Topic Design

The MQTT topic structure was designed using a hierarchical naming convention: panel/{id}/{data_type} for sensor data and panel/{id}/control/{action} for commands. This structure allows the backend to subscribe to all panels using a single wildcard subscription (panel/#), while remaining scalable. Furthermore, adding a new panel requires no changes to the backend or broker configuration.

The following tables list all topics used in the system:

Topics published by ESP32:

Table 4.1 Published Topic Specification

| Topic | Frequency | Payload | Example |
| --- | --- | --- | --- |
| panel/1/temperature | Every 10 seconds | Float | 34.9 |
| panel/1/humidity | Every 10 seconds | Float | 62.0 |
| panel/1/door_status | On state change | String | SECURE, OPEN, GRANTED, TIMEOUT, WRONG_CODE |
| panel/1/fan_status | On state change | String | ON, OFF |
| panel/1/alarm_status | On state change | String | NORMAL, CRITICAL_TEMP, TIMEOUT, WRONG_CODE, TEMP_ACKED |

Topics subscribed by ESP32 (commands from dashboard):

Table 4.2 Subscripted Topic Specification

| Topic | Expected Payload | Action |
| --- | --- | --- |
| panel/1/control/reset | RESET | Executes performFullSystemReset() |

The temperature state machine automatically controls the fan based on hysteresis thresholds (ON at 45°C, OFF at 38°C), thus making manual fan control unnecessary. The reset topic is the only command exposed to the dashboard.

## API Design

The FastAPI backend provides seven REST API endpoints for the React dashboard. All endpoints use JSON responses except the export endpoints, which return CSV files through streaming.

Table 4.3 RESTAPI Specification

| Endpoint | Method | Purpose |
| --- | --- | --- |
| /panels | GET | List all 10 panels with current state |
| /panels/{panel_id} | GET | One panel's current state |
| /panels/{panel_id}/history?hours=24 | GET | Historical sensor readings for a panel |
| /alerts | GET | Alert log (optionally filter ?unacknowledged=true) |
| /panels/{panel_id}/control/reset | POST | Send reset command to ESP32 via MQTT |
| /panels/{panel_id}/export/readings?from_date=YYYYMMDD | GET | Download sensor readings as CSV |
| /panels/{panel_id}/export/alerts?from_date=YYYYMMDD | GET | Download alerts as CSV |

The first four endpoints read from PostgreSQL and return JSON. The two export endpoints query PostgreSQL and stream the results as downloadable CSV files. The reset endpoint is the only POST, it is also the only endpoint that publishes to MQTT rather than reading from the database.

The export endpoints use StreamingResponse to send CSV data in chunks rather than building the entire file in memory. This keeps memory usage flat regardless of dataset size, avoiding resource contention with other services running on the same machine.

Interactive API documentation is automatically generated by FastAPI via Swagger UI, accessible at http://192.168.100.2:8000/docs. This was used throughout development to test each endpoint before building the React frontend, as shown in Figure 4.4.

Figure 4.4 FastAPI Swagger UI displaying all REST API endpoints

## Frontend Design

The React dashboard was designed as a single-page application with three main areas: a title bar, a sidebar, and a detail panel. The layout is illustrated in Figure 4.5.

The title bar spans full width and displays the project name, the count of online panels (e.g., "1/10 Online"), and a live clock showing the current date and time.

The sidebar lists all 10 panels with a green or red status dot indicating online or offline status. Online detection is based on a 15-second threshold on the last_seen timestamp - if a panel has not reported within 15 seconds, it is considered offline. Clicking a panel loads its details in the main content area.

The detail panel displays the selected panel's information in two columns: a Chart.js line chart showing 30 days of temperature and humidity history on the left, and colour-coded status indicators on the right. Status fields use colour coding aligned to the ESP32 firmware thresholds:

Table 4.4 Sensor Threshold Colour Specification

| Field | Green (normal) | Amber (warning) | Red (critical) |
| --- | --- | --- | --- |
| Temperature | Below 45°C | 45 - 74°C | ≥ 75°C |
| Door | SECURE, GRANTED | OPEN | TIMEOUT, WRONG_CODE |
| Fan | OFF | ON | N.A |
| Alarm | NORMAL | TIMEOUT, TEMP_ACKED | CRITICAL_TEMP, WRONG_CODE |

Two action buttons are available: a Logs button opens a modal for exporting sensor readings or alerts as CSV, and a Reboot button sends a reset command to the panel via MQTT.

An error banner appears automatically below the title bar when the backend is unreachable, displaying "Backend unreachable - displaying last known data." The banner disappears when connectivity is restored.

The initial layout was hand-drawn wireframes during the initial design phase:

Figure 4.5 Initial wireframe sketch

The wireframes were refined during development. The separate temperature and humidity charts were combined into a single dual-line chart to improve readability, and a title bar was added to display the online panel count and a live clock. The final dashboard implementation is shown in Figure 4.6.

Figure 4.6 Final dashboard implementation

The component hierarchy follows a Stateful/Stateless pattern:

Table 4.5 Component Hierarchy Specification

| Component | Role | Data responsibility |
| --- | --- | --- |
| App.jsx | Stateful | Owns panels list, clock, connection state |
| Sidebar.jsx | Stateless | Receives panels, renders list |
| PanelDetail.jsx | Stateful | Fetches panel detail and history |
| StatusInfo.jsx | Stateless | Receives panel data, applies colour coding |
| SensorChart.jsx | Stateless | Receives history data, renders chart |
| LogsModal.jsx | Stateless | Receives panel ID, handles CSV export |

Stateful components manage data fetching and state, whereas stateless components receive data via props and handle rendering only.

## ESP32 Firmware Design

The ESP32 firmware manages four core functions: sensor monitoring, automated fan control, door access control, and alarm management. The firmware was developed using Arduino IDE with the PubSubClient library for MQTT communication.

Hardware Configuration

The physical prototype uses an ESP32-WROOM-32E module mounted on an expansion board. The GPIO pin allocation is shown in the table 4.6 below. The ESP32 module, expansion board, and circuit schematic are shown in Figures 4.7, 4.8, and 4.9.

GPIO PIN ALLOCATION

Table 4.6 GPIO Pin Specification

| GPIO Pin | Component | Function |
| --- | --- | --- |
| GPIO 2 | DHT11 | Temperature data |
| GPIO 17 | Reed Switch | Door detection |
| GPIO 18 | Push Button | System reset |
| GPIO 19 | Buzzer | Audio alarm |
| GPIO 21, 22 | LCD 1602 | I2C SDA/SCL |
| GPIO 23 | DC Motor | Fan control |
| GPIO 15,12,14,27 | Keypad Rows | Row scanning |
| GPIO 32,33,4,13 | Keypad Cols | Column detection |

Figure 4.7 ESP32-WROOM-32E

Note. From WiFi Bluetooth Module ESP32-WROOM-32E 4MB 8MB 16MB Flash, by Kuriosity, n.d.-b (Retrieved May 19, 2026, from https://kuriosity.sg/products/wifi-bluetooth-module-esp32-wroom-32e-4mb-16mb-flash). Image sourced from Kuriosity.

Figure 4.8 ESP32 expansion board

Note. From ESP32 Expansion Board, by Kuriosity, n.d.-a (Retrieved May 19, 2026, from https://kuriosity.sg/products/esp32-expansion-board). Image sourced from Kuriosity.

Figure 4.9 Circuit schematic diagram

Temperature State Machine

Fan control is managed by a four-state hysteresis-based state machine. The complete firmware logic, including temperature management, door access control, and alarm handling, is illustrated in Figure 4.10.

Table 4.7 Temperature Status Specification

| State | Condition | Fan | Buzzer |
| --- | --- | --- | --- |
| NORMAL | Below 45°C | OFF | OFF |
| COOLING | 45°C - 74°C | ON | OFF |
| CRITICAL | ≥ 75°C (unacknowledged) | ON | ON |
| ACKNOWLEDGED | ≥ 75°C (acknowledged by technician) | ON | OFF |

Hysteresis prevents rapid toggling: the fan turns ON at 45°C but does not turn OFF until temperature drops below 38°C. This 7°C gap ensures the fan does not repeatedly switch on and off near the threshold.

Figure 4.10 ESP32 firmware flow diagram.

Door Access Control

When the door opens (reed switch circuit break), a 60-second countdown begins. The user must enter the correct password on the keypad within this window. Correct entry grants access. If 60 seconds elapse without entry, a DOOR_TIMEOUT alarm triggers. Three incorrect attempts trigger a SECURITY_BREACH alarm.

Alert Triggers

Three conditions trigger alerts:

Table 4.8 Condition Specification

| Condition | Alert Type | Severity |
| --- | --- | --- |
| Temperature ≥ 75°C | HIGH_TEMP | CRITICAL |
| Door open 60 seconds without password | DOOR_TIMEOUT | WARNING |
| Three incorrect password attempts | SECURITY_BREACH | CRITICAL |

Physical vs Remote Control:

Table 4.9 User Interface Specification for Physical and Remote System Controls

| Action | Physical (on-site) | Remote (dashboard) |
| --- | --- | --- |
| System reset | 3-second hold of reset button | POST to /panels/{id}/control/reset |
| Acknowledge temperature alarm | Double-press reset button | Not available |
| Password entry | Keypad input | Not available |

## Summary

This section described the design of the monitoring system across all layers of the system architecture. It follows a layered structure with the ESP32 at the perception layer, Mosquitto as the message broker, FastAPI as the backend, PostgreSQL as the database storage, and React as the frontend. Data flows inbound from sensors to the dashboard and outbound from the dashboard to the ESP32 device for control commands. The database separates current state from historical logs using a three-table schema. MQTT topics follow a hierarchical naming convention for scalability. Seven REST API endpoints serve the frontend, with StreamingResponse used for efficient CSV log exports. The frontend follows a stateful/stateless component pattern with colour-coded status indicators matching firmware thresholds. The ESP32 firmware manages sensor monitoring, hysteresis-based fan control, door access authentication, and three alert trigger conditions.

# Development

## Introduction

This section details the development process of the monitoring system, following the phased approach outlined in Section 1.6. Each step built upon the previous parts, with completion criteria verified before progressing. The development environment consisted of a Windows 11 PC, Arduino IDE for ESP32 firmware, Python 3.14.4 with a virtual environment for the backend, and Node.js v24 with Vite for the React frontend.

## ESP32 Firmware Development

The ESP32 firmware was the first component developed, as all subsequent steps depended on a working sensor node publishing data. The firmware was written in C++ using Arduino IDE with the PubSubClient library (v2.8.0 by Nick O'Leary) for MQTT communication.

Initial prototyping used Adafruit IO as the MQTT platform. During this phase, the core functionality was established: DHT11 sensor reading, LCD display output, keypad password entry, reed switch door detection, buzzer alarm, and DC motor fan control via PN2222 transistor switching.

After migrating to the self-hosted Mosquitto broker, the firmware was updated to use PubSubClient instead of the Adafruit MQTT library. The MQTT topic structure was changed from Adafruit's flat feed format to a hierarchical panel/{id}/{data_type} convention. Connection credentials were separated into a credentials.h file to keep sensitive data out of the main sketch.

A callback function (mqttCallback) was added to handle incoming MQTT messages, enabling two-way communication. The ESP32 subscribes to control topics on connection and executes commands when messages arrive, currently limited to the system reset command. Refer to Figure 5.1 for the assembled ESP32 Prototype.

Figure 5.1 ESP32 Prototype

## Mosquitto Broker Configuration

Mosquitto was installed on the development PC (Windows 11, IP: 192.168.100.2) and configured with Level 2 security, requiring username and password authentication for all connections. Two MQTT users were created using the mosquitto_passwd utility: esp32_panel for the ESP32 device and backend_server for the FastAPI backend.

The broker configuration file (mosquitto.conf) was set to listen on port 1883 on all network interfaces, enabling connections from both the local machine and the ESP32 over Wi-Fi. Logging and message persistence were enabled for debugging and data reliability. A Windows Firewall rule was added to allow inbound traffic on port 1883.

## Backend Development Environment

A Python virtual environment was created in the FYP_Backend directory to isolate project dependencies. The following packages were installed via pip:

Table 5.1 Backend Package Specification

| Package | Purpose |
| --- | --- |
| FastAPI | Web Framework for RESTAPI |
| uvicorn | Asynchronous Server Gateway Interface (ASGI) to run FastAPI on port: 8000 |
| Paho-MQTT | Client library to establish connection between MQTT and FastAPI |
| Psycopg2-binary | PostgreSQL database connector for Python |

## MQTT Subscriber (Proof of Concept)

Before building the full backend, a standalone script (mqtt_subscriber.py) was created to verify MQTT communication between the ESP32 and the development PC. The script connected to Mosquitto as the backend_server user, subscribed to the panel/# wildcard topic, and printed incoming messages to the console.

This step confirmed that the ESP32 was successfully publishing sensor data and that the Mosquitto broker was routing messages correctly. Once verified, the script was retired and its functionality was incorporated into main.py in the next step.

## PostgreSQL Database Implementation

PostgreSQL Ver18.3 was installed on the development PC and a database named fyp_monitoring was created. Three tables were implemented following the schema designed in Section 4.4 at Figure 4.3 Entity Relationship Diagram.

The panels table was seeded with 10 rows (Panel 1 to Panel 10), each with a unique panel_id, name, and location. Panel 1 represents the physical ESP32 device, while Panels 2 to 10 were reserved for simulated data.

The sensor_readings and alerts tables were created with foreign key constraints referencing panels.panel_id, enforcing referential integrity across the schema.

Database connectivity was verified using the psql command-line tool to confirm table creation, row insertion, and foreign key relationships.

## FastAPI Backend Development

The FastAPI backend (main.py) was the main central component for connecting the MQTT broker, the PostgreSQL database, and the React frontend. The file was structured into seven logical region parts:

Table 5.2 Main.py Fastapi script Region parts

| Region | Purpose |
| --- | --- |
| Imports | All external dependencies |
| Configuration | Constants for MQTT, database, and alert thresholds |
| Database helper | get_db_connection() function providing a fresh connection per caller |
| MQTT callbacks | on_connect (subscribes to topics) and on_message (writes to database) |
| Lifespan hook | Starts MQTT background thread at application startup |
| REST endpoints | Seven API endpoints serving the React frontend |
| Direct run | Convenience shortcut to run via python main.py |

Threading model: The application runs two threads within a single process. The main thread, managed by uvicorn, handles HTTP requests from the React frontend. A background thread, started by paho-mqtt's loop_start(), handles incoming MQTT messages. Both threads access PostgreSQL through separate database connections, avoiding shared state.

MQTT callbacks: The on_connect callback subscribes to the panel/# wildcard topic whenever a connection to Mosquitto is established. This pattern ensure automatic re-subscription is done whenever a connection drops and reconnects. The on_message callback parses each incoming message, performs an INSERT into sensor_readings, an UPDATE to the panels table, and checks alert conditions before inserting into the alerts table if thresholds are exceeded.

Database connection pattern: The get_db_connection() helper returns a fresh PostgreSQL connection each time it is called. Each caller receives its own connection, preventing cross-contamination between the MQTT thread and the HTTP thread.

REST endpoints: Five endpoints were implemented in Step 4 (GET panels, GET panel by ID, GET history, GET alerts, POST reset). Two CSV export endpoints were added in Step 5 using FastAPI's StreamingResponse to deliver data in chunks without loading the full dataset into memory.

CORS middleware: Cross-Origin Resource Sharing (CORS) middleware was added to allow the React development server (running on localhost:5173) to make requests to the FastAPI backend (running on port 8000). Without this, the browser would block all API calls from the frontend.

## React Frontend Development

The React frontend was scaffolded using Vite and developed as a single-page application. Components were built incrementally, with each sub-step adding functionality to the dashboard.

Project setup: The Vite build tool was used to create the React project in the fyp-frontend directory. The following npm packages were installed:

Table 5.3 npm Package Specification

| Package | Purpose |
| --- | --- |
| chart.js | Charting library for line charts |
| react-chartjs-2 | React wrapper for Chart.js |
| chartjs-adapter-date-fns | Date formatting adapter for the chart time axis |
| date-fns | Date utility library required by the adapter |

Service layer: All API calls to FastAPI were centralised in a single file (api.js) containing seven functions, one per endpoint. This separation kept fetch logic out of the UI components, making them easier to maintain and test.

Component build order: Components were developed in a logical sequence, each building on the previous:

Table 5.4 Frontend Component Specification

| Order | Component | Purpose |
| --- | --- | --- |
| 1 | api.js | Service layer with all fetch functions (RESTAPI) |
| 2 | Sidebar.jsx | Panel list with online/offline status dots |
| 3 | PanelDetail.jsx | Main content area with panel header and action buttons |
| 4 | StatusInfo.jsx | Right column displaying colour-coded status fields |
| 5 | SensorChart.jsx | Combined both temperature and humidity line chart (30 days) |
| 6 | LogsModal.jsx | Export modal with dropdown and date input for CSV Log download |
| 7 | App.jsx | Title bar, error banner, loading states, responsive layout |

Key implementation details:

Online/offline detection uses a 15-second threshold on the last_seen timestamp rather than a database status column. If a panel has not reported within 15 seconds, it is displayed as offline.

The sensor chart loads 30 days of historical data once per panel selection using fetchHistory(). Live status data is polled every 5 seconds using fetchPanel(). These two data sources are loaded in parallel using Promise.all() to minimise loading time when switching between panels.

The error banner in App.jsx detects when the FastAPI backend is unreachable by wrapping the polling function in a try/catch block. When a fetch fails, the banner appears automatically. When connectivity is restored on the next successful poll, the banner disappears without manual intervention.

## Summary

This section detailed the development process across seven steps, from ESP32 firmware prototyping to the completed React dashboard. Each step was built and tested independently before progressing to the next, reducing integration risk and error. The Adafruit IO to Mosquitto migration early in the project informed the technology choices for the remainder of the development. The final system consists of ESP32 firmware, a Mosquitto broker, a FastAPI backend with MQTT subscriber and REST API, a PostgreSQL database, and a React frontend, all running on a local network.

# Testing

## Introduction

This section presents the testing conducted on the monitoring system to verify that it meets the requirements defined in Section 3. Testing was performed manually across the full system pipeline, covering functional testing of individual features and integration testing across all components. Each test case is mapped to a specific requirement, with results and evidence documented in the following sections.

## Test plan

The following test plan maps each requirement from Section 3 to a specific test case with expected results.

Table 6.1 Functional Requirements Test Plan

| Test ID | Req ID | Test Description | Expected Result |
| --- | --- | --- | --- |
| T1 | FR1 | Power on ESP32, observe serial monitor for sensor readings | Temperature and humidity values printed every 10 seconds |
| T2 | FR2 | Check Mosquitto verbose log while ESP32 is running | Messages appear on panel/1/temperature and panel/1/humidity topics |
| T3 | FR3 | Allow temperature to rise above 45°C, then cool below 38°C | ESP32 LCD display: Fan turns ON at 45°C, remains ON until temperature drops below 38°C |
| T4 | FR4 | Allow temperature to exceed 75°C (or simulate via sensor) | ESP32 LCD display: Alarm Temp:”Temperature”C CRITICA |
| T5 | FR5 | Open the enclosure door | Reed switch detects state change, door_status message published to MQTT |
| T6 | FR6 | Open door and wait 60 seconds without entering password | ESP32 LCD display: DOOR_TIMEOUT alarm triggers |
| T7 | FR7 | Open door and enter incorrect password three times | ESP32 LCD display: SECURITY_BREACH alarm triggers |
| T8 | FR8 | Query PostgreSQL after ESP32 has been running for 1 minute | sensor_readings table contains rows; panels table shows current values |
| T9 | FR9 | Open dashboard and count panels in sidebar | All 10 panels listed (Panel 1 to Panel 10) |
| T10 | FR10 | Open dashboard in browser, verify all panels visible | Sidebar displays Panels 1 to 10 with status dots |
| T11 | FR11 | Select Panel 1 on dashboard, check chart area | Chart.js line chart displays temperature and humidity data up to 30 days |
| T12 | FR12 | Click Reboot button on dashboard, confirm dialog, observe ESP32 | ESP32 serial monitor shows "FULL SYSTEM RESET" executed |
| T13 | FR13a | Click Logs button, select Sensor Readings, enter date, download | CSV file downloads with correct sensor data |
| T14 | FR13b | Click Logs button, select Alerts, enter date, download | CSV file downloads with correct alert data |
| T15 | FR14 | Observe status fields when temperature enters cooling range (45 to 74°C) | Temperature field displays amber colour coding |
| T16 | FR15 | Check sidebar with ESP32 powered on, then power off ESP32 | Panel 1 status shows green dot when online, red dot after 15 seconds panel offline status. |

Table 6.2 Non - Functional Requirements Test Plan

| Test ID | Req ID | Test Description | Expected Result |
| --- | --- | --- | --- |
| T17 | NFR1 | Publish new sensor data from ESP32, time until dashboard updates | Dashboard reflects new data within 10 seconds |
| T18 | NFR2 | Verify all 10 panels are accessible on the dashboard | All 10 panels selectable; Panel 1 (physical) displays live data, Panels 2 to 10 (simulated) accessible with no data available |
| T19 | NFR3 | Stop Mosquitto broker while FastAPI is running, then query dashboard | Dashboard continues to display last-known panel data |
| T20 | NFR4 | Attempt MQTT connection without credentials (incorrect password) | Connection rejected by Mosquitto |
| T21 | NFR5 | Stop FastAPI backend, observe dashboard | Error banner appears: "Backend unreachable" |
| T22 | NFR6 | Trigger cooling state (45°C+), observe dashboard colour coding | Temperature field changes from green to amber |
| T23 | NFR7 | Confirm all services run on local network only | All connections use 192.168.100.x addresses, no external traffic |
| T24 | NFR8 | Resize browser window to 768px width | Layout adjusts, sidebar stacks above content |

## Results

All test cases were conducted on the local development network with full the system running (Mosquitto, FastAPI, PostgreSQL, and React). Panel 1 (physical ESP32) was used for all hardware-related tests. Results are summarised in the following tables.

Table 6.3 Functional Requirements Test Results

| Test ID | Req ID | Result | Evidence |
| --- | --- | --- | --- |
| T1 | FR1 | Pass | Figure 6.1 Serial Monitoring for Sensor Readings |
| T2 | FR2 | Pass | Figure 6.2 Mosquitto MQTT verbose log |
| T3 | FR3 | Pass | Figure 6.3 ESP32 LCD: fan ON, Cooling state Figure 6.4 ESP32 LCD: Normal state |
| T4 | FR4 | Pass | Figure 6.5 ESP32 LCD: critical temperature alarm |
| T5 | FR5 | Pass | Figure 6.6 Mosquitto MQTT: door state change detected |
| T6 | FR6 | Pass | Figure 6.7 ESP32 LCD: TIMEOUT alarm |
| T7 | FR7 | Pass | Figure 6.8 ESP32 LCD: Incorrect Password Alarm. |
| T8 | FR8 | Pass | Figure 6.9 PostgreSQL query: sensor_readings and panels data |
| T9 | FR9 | Pass | Figure 6.10 Dashboard: all 10 panels listed in sidebar with status colour |
| T10 | FR10 | Pass | Refer to Figure 6.10 Dashboard: all 10 panels listed in sidebar with Panel status. |
| T11 | FR11 | Pass | Figure 6.11 Dashboard: temperature and humidity chart |
| T12 | FR12 | Pass | Figure 6.12 ESP32 LCD: full system reset. |
| T13 | FR13a | Pass | Figure 6.13 Export Logs Modal: Sensor Readings Figure 6.14 CSV export: sensor readings data |
| T14 | FR13b | Pass | Figure 6.15 Export Logs Modal: Alerts Figure 6.16 CSV export: alert records |
| T15 | FR14 | Pass | Figure 6.17 Dashboard: amber colour-coded temperature status at Cooling Mode. |
| T16 | FR15 | Pass | Figure 6.18 Dashboard: green dot (online) Figure 6.19 Dashboard: red dot (offline) |

Table 6.4 Non - Functional Requirements Test Results

| Test ID | Req ID | Result | Evidence |
| --- | --- | --- | --- |
| T17 | NFR1 | Pass | Figure 6.20 Dashboard: data updated within 10 seconds |
| T18 | NFR2 | Pass | Figure 6.21 Dashboard: Selectable Panels, online display data, offline with no data |
| T19 | NFR3 | Pass | Figure 6.22 Dashboard: last-known data with MQTT stopped |
| T20 | NFR4 | Pass | Figure 6.23 Mosquitto log: connection rejected (wrong credentials) |
| T21 | NFR5 | Pass | Figure 6.24 Dashboard: error banner displayed |
| T22 | NFR6 | Pass | Refer to Figure 6.17. Dashboard: amber colour-coded temperature status at Cooling Mode. |
| T23 | NFR7 | Pass | Figure 6.25 Dashboard Access via local network address |
| T24 | NFR8 | Pass | Figure 6.26 Dashboard: responsive layout at 768px |

## Known Limitations

The following limitations were identified during testing:

- Fan status remains NULL in the database until the temperature crosses the 45°C threshold for the first time. This is expected behaviour as the fan state machine only activates above the threshold.

- Door, fan, and alarm status columns in the panels table start as NULL on system startup. Values populate on the first state change event or when the ESP32 is rebooted while the backend is running.

- The critical temperature threshold (75°C) was difficult to trigger consistently during testing due to the limitations of the DHT11 sensor in a home environment.

- No automated testing framework was used. All tests were conducted manually. Automated testing (pytest or postman for backend, React Testing Library for frontend) is recommended as future work.

- Alert CSV export requires at least one alert record to exist in the database. If no alerts have been triggered, the export returns an empty file.

- Simulated panels (2 to 10) have no sensor data as the simulator script was not developed within the project scope.

## Summary

This section presented the summary of the testing strategy, test plan, and results for the monitoring system. A total of 24 test cases were defined covering 15 functional requirements and 8 non-functional requirements. All test cases passed successfully. Evidence was provided through screenshots of the serial monitor, Mosquitto logs, PostgreSQL queries, and the React dashboard. Known limitations were documented for transparency and are addressed in the future work recommendations in Section 7.

# Conclusion

## Recommendations

This section evaluates the project outcomes against the original aim and objectives, reviews the risks encountered, reflects on the project management approach, presents recommendations for future work/improvement, and concludes with a final reflection on the project.

## Aim & Objectives

The aim of this project was to develop a multi-panel electronics enclosure monitoring system with automated cooling, alert notifications, and a web-based dashboard for real-time monitoring and historical trend analysis.

The following table evaluates each objective against its resulted outcome:

Table 7.1 Objectives Evaluation

| # | Objective | Status | Evidence |
| --- | --- | --- | --- |
| 1 | Research existing IoT monitoring solutions and evaluate suitable technologies | Met | Section 2: Literature Review |
| 2 | Design a system architecture using MQTT | Met | Section 4.2: System Architecture |
| 3 | Develop ESP32 firmware with sensors, fan control, and MQTT | Met | Section 5.2: ESP32 Firmware Development |
| 4 | Configure a self-hosted Mosquitto broker with authentication | Met | Section 5.3: Mosquitto Broker Configuration |
| 5 | Implement a FastAPI backend with MQTT subscriber and PostgreSQL | Met | Section 5.7: FastAPI Backend Development |
| 6 | Design and fill a PostgreSQL database | Met | Section 5.6: PostgreSQL Database Implementation |
| 7 | Build a React web dashboard with Chart.js | Met | Section 5.8: React Frontend Development |
| 8 | Implement an alert system for critical state conditions | Met | Section 6: Testing (T4, T6, T7) |
| 9 | Conduct end-to-end testing and evaluate against requirements | Met | Section 6: Testing (24 test cases, all passed) |

All nine objectives were met. The original PID included additional objectives for RFID access control and message alert notifications. These were deferred to future work due to time constraints and scope prioritisation, as documented in Section 3.5.

## Risks and outcomes

The risk log defined in Section 1.4 identified eight risks. The following table summarises the outcomes:

Table 7.2 Risk Outcomes

| # | Risk | Outcome | Discussion |
| --- | --- | --- | --- |
| 1 | Hardware component failure | Did not occur | The single physical ESP32 operated reliably throughout development and testing |
| 2 | Data loss | Did not occur | GitHub version control and PostgreSQL persistence prevented data loss |
| 3 | Scope creep | Mitigated | RFID, message alerts, and predictive maintenance prompts were deferred early to maintain focus on core functionality |
| 4 | Component integration complexity | Mitigated | The phased development approach allowed each component to be tested independently before integration |
| 5 | Network connectivity failure | Occurred, resolved | The ESP32 could not reach the PC when the phone was on mobile data instead of Wi-Fi. Resolved by switching the phone to Wi-Fi mode and documenting the network topology |
| 6 | RFID module compatibility | Not applicable | RFID was deferred to future work |
| 7 | Personal illness or commitments | Occurred, resolved | Buffer time in the schedule accommodated the disruption |
| 8 | Network failure during demonstration | Pending | Video demonstration recorded as fallback (see Appendix G) |

Risk 5 was the most significant technical challenge encountered. The root cause was the mobile hotspot routing behaviour: when the phone used mobile data, traffic was routed externally rather than staying on the local subnet, thus causing ESP32 connection to be IP address incorrectly routed. Switching to Wi-Fi mode kept all traffic on the 192.168.100.x network. This experience reinforced the importance of documenting network topology and testing connectivity early.

## Project Management

The phased development approach proved effective. Dividing the project into seven sequential steps with clear completion criteria thus reduced integration risk and provided trackable progress at each stage. Each step built upon the previous, ensuring a working system at every checkpoint.

The Gantt chart (Appendix A) provided an initial timeline for the project. While the overall sequence of tasks was followed, some phases took longer than planned, particularly the React frontend development which required learning new concepts such as state management and component architecture. Buffer time allocated in the schedule accommodated these delays.

Supervisor meetings with Dr. Loo Poh Kok provided regular guidance and progress reviews. Version control through GitHub ensured code was backed up and changes were tracked throughout the project.

If the project were to be repeated, the following adjustments would be made: allocating more time for the report writing phase, implementing automated testing earlier in the development process, and building a panel simulator script sooner to generate test data for Panels 2 to 10.

## Future work recommendation

The following enhancements are recommended for future works and development:

| Category | Enhancement | Description |
| --- | --- | --- |
| Deployment | Docker containerisation | Package all services into containers with docker that compose for single command deployment |
| Deployment | Cloud deployment | Deploy to a VPS (Linode/AWS) for remote access outside the local network |
| Security | TLS/SSL encryption | Encrypt MQTT traffic on port 8883 to secure data in transit |
| Security | User authentication | Add login system for the dashboard to prevent unauthorised access and actions |
| Security | Hashed Mosquitto passwords | Replace plaintext password file with hashed credentials |
| MQTT | MQTT over WebSockets | Enable the React frontend to subscribe directly to Mosquitto for real-time push updates |
| Database | Alert recovery logging (Error Recovery Logs) | Record when alert conditions are resolved, not just when they are triggered |
| Database | Data retention policy | Archive or delete old sensor readings to manage database growth |
| Database | Connection pooling | Replace per-request connections with a connection pool for improved performance |
| Frontend | Notification system | Email or SMS alerts via SendGrid/Twilio for critical conditions |
| Hardware | Upgrade DHT11 to DHT22 | The Threshold temperature for DTH11is not suitable for this monitoring system, as its specification is only from 0 to 50 Degree. Where else DHT 22 is from -40 to 80 Degree |
| Hardware | RFID access control | Card scan combined with passcode entry to authenticate individual personnel |
| Hardware | OTA firmware updates | Enable over-the-air firmware updates without physical access to the ESP32 |
| Testing | Automated testing | pytest or Postman for backend API testing, React Testing Library for frontend |
| Analytics | Predictive maintenance prompts | Detect recurring patterns and prompt operators to schedule maintenance through the data trend |

## Conclusion

# References

Banaamah, A.M. and Ahmad, I. (2024) 'Internet of Things: a comprehensive overview, architectures, applications, simulation tools, challenges and future directions', Discover Internet of Things, 4(1). Available at: https://doi.org/10.1007/s43926-024-00084-3

Eclipse Foundation (no date) Eclipse Mosquitto: An Open Source MQTT Broker. Available at: https://mosquitto.org/ (Accessed: May 2026).

Espressif Systems (2024) ESP32 Series Datasheet, version 5.2. Available at: https://www.espressif.com/sites/default/files/documentation/esp32_datasheet_en.pdf (Accessed: May 2026).

FastAPI (no date) FastAPI Documentation. Available at: https://fastapi.tiangolo.com/ (Accessed: May 2026).

Kuriosity (no date a) WiFi Bluetooth Module ESP32-WROOM-32E 4MB 8MB 16MB Flash. Available at: https://kuriosity.sg/products/wifi-bluetooth-module-esp32-wroom-32e-4mb-16mb-flash (Accessed: May 2026).

Kuriosity (no date b) ESP32 Expansion Board. Available at: https://kuriosity.sg/products/esp32-expansion-board (Accessed: May 2026).

Lv, Y.-G., Chu, W.-X. and Wang, Q.-W. (2022) 'Thermal management systems for electronics using in deep downhole environment: A review', International Communications in Heat and Mass Transfer, 139, 106450. Available at: https://www.sciencedirect.com/science/article/abs/pii/S0735193322005723

mqtt.org (no date) MQTT: The Standard for IoT Messaging. Available at: https://mqtt.org/ (Accessed: May 2026).

OASIS (2014) MQTT Version 3.1.1, OASIS Standard. Available at: https://docs.oasis-open.org/mqtt/mqtt/v3.1.1/os/mqtt-v3.1.1-os.html (Accessed: May 2026).

PostgreSQL (no date) PostgreSQL: The World's Most Advanced Open Source Relational Database. Available at: https://www.postgresql.org/ (Accessed: May 2026).

React (no date) React Documentation. Available at: https://react.dev/ (Accessed: May 2026).

# Appendices – on a new page

# Appendix A: Project Initiation Document (include Gantt Chart)

Copy & paste into this document

# Appendix B : Ethics Certificate – on a new page

You might also have as appendices your requirements specification, any UML diagrams etc